How to Install this Intake System:

GT-Turbo Cold-Air Intake System
[image: image1.jpg]



Installation is very simple and easy. If you should have any questions, please contact us at 909-924-3201, 9am - 5pm, M-F, PST and we will be glad to help.

Your intake kit should have 4 parts: 
1 - Metal intake housing, 
2 - Air Force One basket-type filter element, 
3 - Filter element clamp &
4 - Press-on weather-seal gasket. 

If any parts are missing, please contact us at the number above before installing your intake system.
The filter is a permanent-type, reusable element and should be cleaned periodically (every 7,500 miles under normal use, more often in dusty conditions.) Use the BTG™ Blue-Oil Reusable Filter Recharge Kit, (part # PT1028BTG) available via your selling dealer or order online at www.cruisermotorsports.com.

1) Disconnect the Negative Battery Cable (-).
2) Loosen the intake tube clamp on the stock air box lid, then unclamp and remove the lid.

3) Remove the air box, leaving the base in place.

4) Install the Turbo air box assembly and clamp in place with the factory clamps.

5) Place the OEM intake tube onto the new Turbo air box and secure with the OEM clamp.

6) Loosen supplied clamp on the Turbo air-filter element and slip the filter into place inside the new air box, then tighten clamp.

7) Press the supplied weather-seal gasket into place on three sides, leaving the side facing the fender located cold-air-inlet clear. (Cutting the hard plastic channel at 90 degree bends makes installation easier.)

8) Make sure no foreign objects or other debris are in the intake area.

9) Reconnect the battery cable and away you go!
Simple! -- Easy! -- Inexpensive! -- FUN!!!
